SECTION 28 0500 - COMMON WORK RESULTS FOR ELECTRONIC SAFETY AND SECURITY

Maintain

Revise this Section by deleting and inserting text to meet Project-specific requirements.

Maintain Section format, including the UH Master Spec designation and version date in bold in the center columns in the header and footer. Complete the header and footer with Project information.

Designer is required to adhere to the University’s “Electronic Access Control Design Guide” and “Network Infrastructure Design Standards” available in Owner’s Design Guidelines on the Facilities Planning and Construction website, and to “Electronic Safety and Security Design Guide: Electronic Access Control and Intrusion Detection,” “Electronic Safety and Security Design Guide: Surveillance and Call Stations” and “Design Deliverable Checklist: Security” available in IT Facilities Standards on University Information Technology website.

This Section uses the term "Architect" or “Engineer.” Change this term to match that used to identify the design professional as defined in the General and Supplementary Conditions.

Verify that Section titles referenced in this Section are correct for this Project's Specifications; Section titles may have changed.

Delete hidden text after this Section has been edited for the Project.

GENERAL

RELATED DOCUMENTS

Drawings and General Provisions of the Contract, including General and Supplementary Conditions and Division 01 Specification sections, apply to work of this Section.

The Contractor's attention is specifically directed, but not limited, to the following documents for additional requirements:

The current version of the Uniform General Conditions for Construction Contracts, State of Texas, available on the web site of the Texas Facilities Commission.

The University of Houston’s Supplemental General Conditions and Special Conditions for Construction.

SECTION INCLUDES

Administrative and coordination requirements for Electronic Safety and Security scope of work.

PRE-INSTALLATION MEETING

Conduct a pre-installation meeting, in compliance with the requirements of Section 01 3100 “Project Management and Coordination,” with the Contractor, Owner’s Project Manager, UIT Project Manager and EAC and Campus Safety Representatives at the Project Site.

Use the meeting to clarify all requirements (systems, services, distribution methods, etc.), identify responsibilities, and schedule the events necessary for implementation of the Project.

Within two (2) weeks of the initial meeting, provide a written report and project schedule to all participants to document the events and responsibilities associated with the Project.

Meeting Agenda:

Sample agenda: For each pre-installation meeting, edit and add to the list below for the topics to be discussed:

Introductions

Review of Contractor Qualifications

Review of Project Scope

Planned and Emergency Outage Guidelines (Division 00 6000 – OPF)

Emergency Contact Protocol (In event of an accidental cable cut or other incident)

Review of Required Site Observation and Approval Walkthroughs

Underground / Under-slab (prior to concrete pour)

In-wall (Prior to sheet rock)

In-ceiling (Prior to ceiling installation)

Review of Project Submittals – Approved and Pending

Review of Project Close-Out Documentation and Timing.

Action Items for Follow-Up.

REFERENCES

Related Division 28 Sections and the latest revisions and pertinent addenda of the following:

National Electric Code (NEC).

ADA Standards for Accessible Design 28 CFR Part 36.

U.S. Department of Labor Occupational Safety & Health Administration (OSHA).

BICSI Electronic Safety & Security Design Reference Manual.

ANSI/TIA568.1-E - Commercial Building Telecommunications Cabling Standard.

ANSI/TIA568.3-D - Optical Fiber Cabling Components Standard.

ANSI/TIA569-D - Commercial Building Standard for Telecommunications Pathways and Spaces.

ANSI/TIA606-C - Administration Standard for Commercial Telecommunications Infrastructures.

ANSI J-STD-607-C, Commercial Building. Grounding/Bounding Requirements- Joint Standard for Commercial Building Grounding (Earthing) and Bonding Requirements for Telecommunications.

ANSI/TIA758-A - Customer-owned Outside Plant Telecommunications Infrastructure Standard.

International Standards Organization/International Electro technical Commission (ISO/IEC) 11801-6.

Underwriters Laboratories (UL) Cable Certification and Follow up Program

National Electrical Manufacturers Association (NEMA)

American Society for Testing Materials (ASTM)

Acronyms and Abbreviations

ACS		Access Control System

ADA	Americans with Disabilities Act

AMS	Access Management Systems

ANSI	American National Standards Institute

AP		Access Provider

ASTM	American Society for Testing and Materials

AWG	American Wire Gauge

BD		Building Distributor (Main Communications Room, formerly BDF Building Distribution Frame)

BICSI	Building Industry Consulting Services International

CATV	Community antenna television

CC		Cross Connect

CCTV	Closed circuit television

CD		Campus Distributor (formerly MDFMain Distribution Frame, or MC, main cross-connect)

CO-OSP	Customer Owned Outside Plant

DVR	Digital Video Recorder

EAC 	Electronic Access Control

EF		Entrance Facility

EIA		Electronic Industries Alliance

EMI		Electromagnetic Interference

FCC		Federal Communications Commission

FD		Floor Distributor (formerly IDF, Intermediate Distribution Frame)

FPS		Frames per second

HVAC	Heating, Ventilation and Air Conditioning

IEEE	The Institute of Electrical and Electronics Engineers

IDS		Intrusion Detection System

ITNO	Information Technology Network Operations

ISO		International Organization for Standardization

LAN	Local Area Network

Mb/s	Megabits Per Second

NEMA	National Electrical Manufacturers Association

NESC	National Electrical Safety Code

NIC	Not in Contract

NFPA	National Fire Protection Association

NVR	Network Video Recorder

OFCI	Owner Furnished, Contractor Installed

OFOI	Owner Furnished, Owner Installed

PBB		Primary Bonding Busbar

RCDD	Registered Communications Distribution Designer

SBB		Secondary Bonding Busbar

SCS		Structured Cabling System

SMS	Security Management Systems

TBB		Telecommunications Bonding Backbone

TR		Telecommunications Room

TIA		Telecommunications Industry Association

UHPS	University of Houston Public Safety Department

UL		Underwriters Laboratories

UPS		Uninterruptable Power Supply

UTP	Unshielded Twisted-Pair

VMS	Video Management System

WA		Work Area

WAP	Wireless Access Point

Wi-Fi	Wireless Telecommunications defined by IEEE 802.11

ADMINISTRATIVE REQUIREMENTS

Coordination

Work furnished (except as indicated) and installed by the Safety Systems Contractor as specified in Division 28 Sections and as shown in Electrical, Security, and Technology Drawings includes:

Electronic Access Control Systems

Video Surveillance Systems (OFCI)

Intrusion Detection Systems

Emergency Call  Stations s (Call Boxes and Call Towers)

Grounding and Bonding for Electronic Safety and Security Systems

Identification for Electronic Safety and Security Systems

Testing for all Electronic Safety and Security Systems

Coordination with Communications, Electrical, Elevator and Door Hardware Contractors

Coordination with Owner’s EAC and Campus Safety Representatives

Work under this Division that is not in the Contract and will be Owner Furnished, Owner Installed includes:

Include list of Owner furnished, Owner installed items for this Project in subparagraph below.

<include list of OFOI items>

Network Equipment

Work provided by the Electrical Contractor as specified in Division 28 and as shown in Electrical, Technology, and Security Drawings includes:

Conduits and back boxes for Electronic Safety and Security device outlets

Horizontal and vertical gutter boxes for cable management in Security Rooms

Electrical circuits for Security Rooms, Doors with Operators, Emergency Call Boxes and Call Towers.

Bonding backbone and associated busbars

Work provided by the Communications Contractor as specified in Division 28 and as shown in Technology and Security Drawings includes:

IP-based security cabling

Work furnished and installed by others:

Covering of Security Room walls, floor to ceiling, with rigidly fixed, 3/4-inch fire rated plywood, void free and capable of supporting attached connecting hardware; and painting of plywood with two coats of fire retardant paint per Section 27 1100 “Network Facility Fittings.”

WORK RESULTS - DESCRIPTION OF PROJECT

Designer to list the elements that constitute the completed work results in Part 1 of each relevant Division 28 Section.

Administrative Services

Attend vendor and product submittal coordination meetings per each Division 28 section.

Submit shop drawings per each Division 28 section.

Provide As-Built Drawings and test results prior to Project acceptance. Refer to Section 28 0600 “Testing for Electronic Safety and Security.”

Grounding and Bonding for Electronic Safety and Security

Coordinate final layout and installation of the grounding and bonding system with the Electrical Engineer of Record.

Install bonding conductors from the PBB or SBB to all security equipment cabinets, equipment racks, raceway, cable ladder rack, cable tray, sleeves and conduits. Bond all SBBs to the PBB in compliance with Section 28 0526 “Grounding and Bonding for Electronic Safety and Security.”

Bond PBB to building ground in compliance with Section 28 0526 “Grounding and Bonding for Electronic Safety and Security.”

Pathways for Electronic Safety and Security

Provide conduit and other pathways in accordance with Section 28 0528 “Pathways for Electronic Safety and Security.”

Identification for Electronic Safety and Security

Provide cable labeling in compliance with ANSI/TIA606-C - Administration Standard for Commercial Telecommunications Infrastructures and as described in Section 28 0553 “Identification for Electronic Safety and Security.”

Comply with Owner’s preferred administrative labeling standards for cabling and its numerical positions on the termination hardware.

Electronic Safety and Security Equipment Room Fittings

Provide each BD, FD and Security Room with proper equipment per Division 28 Specifications and Drawings.

Electronic Safety and Security Horizontal Cabling

Provide cabling per Sections 28 1300 “Access Control,” 28 1600 “Intrusion Detection” and 28 2300 “Video Surveillance.”

Termination Hardware:

Terminate all electronic safety and security cabling in proper enclosures, back boxes and electronic devices per Drawings and manufacturer standards.

Data Communications Equipment

Data communications equipment will be OFOI.

Inter-connectivity with Other Trades:

Network Interface – Coordinate interconnection of copper cabling provided by Communications Contractor with IP cameras, Emergency Call Boxes and Call Towers, Access Control Panels, etc. Refer to Security and Technology Drawings and Division 27 specifications for details.

Fire Alarm – Provide Copper or fiber connectivity as required for fire alarm panels. Coordinate with Section 28 3100 “Addressable Fire Alarm System.”

Select one or both options in subparagraph below to suit Project.

Elevator Equipment Room – Provide Copper connectivity to elevator equipment room(s). Coordinate with [Section 14 2100 “Electric Traction Elevators] [and] [Section 14 2400 “Hydraulic Elevators].

Commissioning Administration:

Comply with Section 01 9113 “General Commissioning Requirements” for commissioning of the Electronic Safety and Security systems.

Project Meetings

Comply with requirements of Section 01 3100 “Project Management and Coordination” and individual Division 28 Sections.

Schedule a pre-installation meeting to clarify Project requirements (systems, services, distribution methods, etc.), identify responsibilities, and establish key milestones for the Project.

Within two weeks of the initial meeting, provide a written report and project schedule to document the Electronic Safety and Security events and responsibilities associated with the Project.

Provide representation on the commissioning team as required for implementation of the Commissioning Plan. Refer to Section 01 9113 “General Commissioning Requirements.”

SUBMITTAL ADMINISTRATIVE REQUIREMENTS

Follow Submittal Administrative Requirements as stated in Section 01 3300 “Submittal Procedures.” Use electronic format only.

Submit shop drawings, product data, and samples promptly and in appropriate sequence to prevent Project schedule delays.

For renovation work: Update existing documentation so that a complete, consolidated inventory of all Work completed is maintained.

ACTION SUBMITTALS

Comply with requirements of Section 01 3300 “Submittal Procedures.”  Include the following:

Qualifications of Safety Systems Contractor

Show a history of having worked satisfactorily for a minimum of five years on systems of the specified type and size.

Furnish a list of references with specific information describing type of project, involvement in providing equipment and systems, and names and contact information.

Provide manufacturer’s certifications for installation and systems technicians

Product Information:

Include the manufacturer's name, model or catalog numbers, catalog information, technical data sheets, pictures, nameplate data, and test reports as required.

Shop Drawings and Wiring Diagrams:

Provide elementary and interconnection wiring diagrams for communication and signal systems, control systems and equipment assemblies. Identify all terminal points and wiring.

Parts List:

Identify replacement parts recommended by the equipment manufacturer, quantity of parts, unit prices and lead time for each part.

SUBSTITUTION REQUESTS

Comply with requirements of Section 01 2500 “Substitution Procedures.” Substitutions require written approval from the Owner’s Project Manager, UIT Project Manager and EAC and Campus Safety Representatives before proceeding with the Work.

Submit product samples for substituted or end of life replacement products.

QUALITY ASSURANCE

Follow Section 01 4000 “Quality Requirements” and this Section.

Ensure that all on-site installation technicians are manufacturer trained and certified, and that all installation technicians possess manufacturer's certificates for systems used on the project.

Ensure that testing technicians possess manufacturer's certificates of completion for the test equipment used on the Project

Do not subcontract installation of Electronic Safety and Security system cabling, termination or testing without written approval of the Owner’s UIT Project Manager and EAC and Campus Safety Representatives.

Coordinate quality assurance site observations and testing with the Owner’s EAC and Campus Safety Representatives.

Red Line Drawings: Maintain one full-size set of Drawings at the Project Site during working hours with installation progress marked and device labels noted for all Work in Division 28. Ensure that Red Line Drawings are available for examination during construction meetings and field inspections.

DELIVERY, STORAGE, AND HANDLING

Delivery, Storage and Handling Requirements: Comply with Section 01 6000 "Product Requirements."

Temporary Storage: Coordinate with Owner as necessary for temporary secure storage of equipment and materials during Project timeframes.

SPARE PARTS

Provide one spare camera model for each type installed per Section 28 2300 “Video Surveillance.”

Provide spare access control parts per Section 28 1300 “Access Control.”

Provide one spare panic button for every 20 installed per Section 28 1600 “Intrusion Detection.”

Provide one spare IP communications plate for every three installed per Section 28 2600 “Emergency Call Systems.”

WARRANTY

Manufacturer's standard form in which manufacturer agrees to repair or replace all security equipment and cabling components that fail in materials or workmanship within specified warranty period.

Provide parts registration, including date and serial numbers, for all components.

Provide components that are performance matched and warranted by the manufacturer.

Warranty Period: Two years from date of Substantial Completion including parts, labor and testing after replacement.

PRODUCTS

PRODUCTS

Refer to individual Division 28 sections for approved products and schedules.

Provide materials, equipment and accessories that are standard Commercial-Off-The-Shelf (COTS) products of a manufacturer engaged in the manufacture of such products. Use typical commercial designs that comply with the requirements specified and are readily available through manufacturers and authorized distributors.

EQUIPMENT REQUIREMENTS

Where variations from the Contract Documents are requested in accordance with the “Uniform General Conditions,” Section 01 6000 “Product Requirements,” and Section 01 2500 “Substitution Procedures,” the connecting Work and related components shall include, but not be limited to, additions or changes to branch circuits, circuit protective devices, conduits, wire, feeders, controls, panels and installation methods.

LABELING

Refer to Section 28 0553 “Identification for Electronic Safety and Security.”

EXECUTION

SITE CONDITIONS

Use of Site

Refer to Section 01 3100 "Project Management and Coordination."

EXAMINATION

Prior to starting the installation, schedule a walk-through of the Project site with the Owner’s Project Manager, UIT Project Manager and EAC and Campus Safety Representatives to review the installation documentation and verify that construction necessary for the installation has been completed and complies with Division 28 requirements.

Verify liquid-carrying pipes are not installed in or above Security Rooms.

Verify fire-rated plywood backboards are properly installed and painted in accordance with Section 09 9123 “Interior Painting.”

Verify conduit, raceways, and boxes are properly installed.

Verify that Security Rooms are clean and dust free.

Notify the Owner’s Project Manager, UIT Project Manager and EAC and Campus Safety Representatives immediately, and prior to installation, in the event that field conditions do not comply with Division 28 requirements.

Proceed with installation only after non-compliant conditions have been corrected.

PREPARATION

Protection of Surroundings:

Protect surrounding construction in compliance with the requirements of Section 01 5000 “Temporary Facilities & Controls.”

Patch and repair facilities, finishes, and equipment in compliance with Section 01 7300 “Execution.”

Continuation of Service:

All security devices currently in use must remain functional during and after the project unless UHPD has signed off on a demolition plan. Demolition plans for UHPD approval will come with schedules for outages, schedules for the return to service, and notification of any area to remain an active part of the UH campus for visitors, faculty, students, and/or staff. UHPD may request a temporary solution to be coordinated with Campus Safety Systems for system compatibility and coverage at the expense of the project. Temporary outages for the changeover of equipment are excluded so long as the outage will not last more than 4 hours and UHPD has been informed of the outage.

Security Device Outage: Time period between when a security device is no longer in service to when it is returned to service

Security Device: Cameras, Alarms, Card Readers, Emergency Phones, and their incorporated system components including hardware, software, and communication.

Contacts:

Primary Authorizing Authority – UH Chief of Police

Secondary Authorizing Authority – UH Assistant Chief of Police

Campus Safety Systems – UH IT Director

PRODUCT QUALITY ASSURANCE

Verify that all materials, accessories and equipment match the approved submittals.

FIRESTOPPING

Properly fire-stop penetrations through fire barriers used for the placement of security cabling. Refer to Section 07 8413 “Penetration Firestopping.”

Coordinate with Electrical Contractor and ensure fire-stopping of security pathway is properly identified and labeled. Laminate and permanently affix to each side of fire or smoke-rated partitions or floor ceiling assemblies, the following information:

Installing contractor's name, address and phone number.

Alpha-numeric unique identifier (floor/penetration - A1)

Name of manufacturer of fire stopping system

Part and model numbers of system and all components

Phone numbers of manufacturer's corporate headquarters in U.S. and local distributor's name and phone number.

CONSTRUCTION WASTE MANAGEMENT

Refer to Section 01 74 19 “Construction Waste Management and Disposal” for requirements.

SYSTEM TESTING AND STARTUP

Refer to Section 28 0600 “Testing for Electronic Safety and Security.”

CLOSEOUT ACTIVITIES

Follow Section 01 7700 “Closeout Procedures” and this Section.

Acceptance is subject to Substantial Completion of all work, successful post-installation testing and completion of the tasks listed below:

Approval of Submittals for Project Record Documents.

Completion of punch list items.

Cleaning of installed work in compliance with Section 01 7700 “Closeout Procedures.”

Completion of Closeout Checklist in compliance with Section 01 7700 “Closeout Procedures.”

Submittals for Project Record

As-Built Drawings: Submit as-built documentation five business days prior to obtaining approval for cutover to any portion of the security system. Submit all documents for review and comments in .rvt, .dwg and .pdf formats.

Final Approved Shop Drawings, including:

Plan and elevations of Security Rooms.

Cable pathway details.

Backbone and horizontal cable locations and cable ID#.

Equipment.

Mac address and assigned IP address.

Block diagrams.

Interface requirements including connector types and pin-outs for all security equipment.

Fabrication drawings for custom built equipment.

Laminated As-Builts: One set of full size final As-Built Drawings. Place copies in the appropriate Security Room.

Equipment Inventory: Four sets of hard copies and one electronic copy of equipment /device inventory data for all cable, terminations and hardware in .xlsx format, listing products furnished, including:

Manufacturer's name and part numbers.

Cable numbers using the Owner's cable numbering standard.

Location and Owner’s label / tagging assignments.

Mac address, cable ID, and IP address of all network devices.

Test Reports: Digital copies in PDF and native format with related Test Result Documentation.

Operation and Maintenance Manuals: Four hard copies and one electronic copy in PDF format.

Warranty

Within 21 days of Substantial Completion of the Project, provide Owner with the Warranty signed by each manufacturer. List in the warranty the Owner and name of the facility, including location, as holder of the warranty.

Operation and Maintenance (O&M) Manuals:

Provide the following identification on the cover: "OPERATION AND MAINTENANCE MANUAL," the name and location of the system, equipment, building, name of Contractor, and contract number.

Include in the manual the names, addresses, and telephone numbers of each subcontractor installing the system or equipment and the local representatives for the system or equipment.

Provide a "Table of Contents" and assemble the manual to conform to the table of contents, with tab sheets placed before instructions covering each subject. Provide instructions that are legible and easily read, with large sheets of drawings folded in.

Include the following:

Internal and interconnecting wiring and control diagrams with data to explain detailed operation and control of the equipment

Control sequence describing start-up, operation, and shutdown

Description of the function of each principal item of equipment

Installation and maintenance instructions

Safety precautions

Diagrams and illustrations

Testing methods

Performance data

Pictorial "exploded" parts list with part numbers. Describe the use of special tools and instruments. Indicate sources of supply, recommended spare parts, and name of servicing organization

Appendix with list of qualified permanent servicing organizations for support of the equipment, including addresses and certified qualifications

TRAINING

Provide training in accordance with Section 01 7900 “Demonstration and Training” and Division 28 Sections.

Develop and submit a training schedule approved by Owner’s UIT Project Manager and EAC and Campus Safety Representatives at least 30 days prior to each planned training session required in each Division 28 Section.

Provide training for the particular equipment or system as required in each associated Division 28 specification.

END OF SECTION 28 0500